Evaluation of Liquefaction Based on Shear Wave Velocity Using Machine Learning

# Shaymaa Kennedy1, Maher Alabbood2, Iman M. Jaafar3

1,3 Basrah University, Basrah, Iraq.

2 West Virginia University, U.S.A. Corresponding author: Shaymaa.kennedy@uobasrah.edu.iq

Abstract – Liquefaction studies use various approaches based on in-situ and laboratory tests. This paper presents research work focused on an investigation to estimate the probability of liquefaction in soils based on data from a regain between Turkey and Iraq. We develop models for the estimation of liquefaction probability using machine learning methods, in particular, RFs, where shear wave velocity becomes the most critical input variable, followed by earthquake magnitude and peak acceleration. The database includes effective vertical stress (σ′v0), soil type, Vs, and seismic parameters of PGA and M, allowing the calculation of liquefaction risk as actual values. The outcomes of the study showed that Random Forest yielded an excellent accuracy of 92.5% in predicting the liquefaction potential of soils with only 20% of the data. The result of this study continues the development of risk assessment methodologies in earthquake-prone areas, which is considered to enhance the resiliency of infrastructures against disaster events.

Keywords: Liquefaction, shear wave velocity, Random Forest, Probability of Liquefaction.

# Introduction

Estimation of the liquefaction potential of soils is one of the important factors in geotechnical engineering, particularly in seismic-prone regions. Liquefaction, occurring due to the rise of pore water pressure within saturated soil during earthquakes, has caused damage to structures and claimed human lives in history. Attempts to assess liquefaction potential based on different methods have been made. Among them, Vs-based approaches have been proposed, using it as an important feature to describe the basic characteristics of soil stiffness and strength.

Numerous investigators have provided various approaches for evaluating liquefaction based on in-situ measurements such as shear wave velocity. [9], [14], [15] [11]. In 1971, following the 1964 Niigata and Alaska earthquakes, Seed and Idriss developed the "simplified method," which determines soil liquefaction potential. Seed et al. (1975a, b, 1983, 1985), [16], improved on this strategy. In the early 2000s, more research was conducted to improve this approach, including studies by [6], [7], [10] and [21].

This study examined liquefaction using field data obtained from Turkey and Iraq. [13] estimating the susceptibility of granular materials to probable liquefaction in Basra using SPT data demonstrated that some locations may experience liquefaction of high supposed earthquake magnitude. Other studies have been done to evaluate liquefaction based on SPT in Halabja, north of Iraq [2] and Marmora Regine [16].

The possibility of liquefaction is determined by analysing shear wave velocity (Vs), a key dynamic characteristic of soils. Vs values can be calculated using a variety of methods, including traditional geophysical techniques such as seismic refraction and reflection, as well as borehole geophysical methods such as cross-hole and down-hole measurements. To calculate liquefaction potential (PL), parameters such as effective vertical stress (σ′v0), soil type, and earthquake characteristics are essential. Nevertheless, machine learning plays a vital role in enhancing the process of liquefaction evaluation, hence reducing time and costs. It offers an effective set of tools for liquefaction assessments that allow scientists and engineers to develop precise, flexible, and adaptable models to evaluate liquefaction susceptibility and mitigate seismic risks.

# Tectonic Features

Turkey, which is situated in the Alpine-Himalayan orogenesis, has been a site of significant quakes across the North Anatolian Fault zone for a long period of time. The Anatolian-Eurasian plate boundary is characterised by Ketin (1948) as an intracontinental right-lateral strike-slip transform fault that covers approximately 1500 km in northern Turkey. The 1999 Izmit, Duzce, and 1967 Mudurnu earthquakes, which severely devastated the study region and its environs, are the most significant earthquakes in the western branch of the NAFZ in the last 60 years. The North Anatolian Fault Zone, which starts with the Karliova triple junction and stretches approximately 1500 km west, is the central part of this earthquake activity. The Marmara region is a part of the North Anatolian Fault (NAF), which extends towards northern Turkey.

It is located along the boundary between the Anatolian Plate and the Eurasian Plate, where the interaction of these plates is highly problematic [16]. The Marmara region has a geology comprising sedimentary, metamorphic, and igneous rocks. The seismic activity surrounding the region involves fault zones such as the North Anatolian Fault and its branches. As a result of its proximity to the Sea of Marmara and the presence of soft sediments in coastal areas, the Marmara region is vulnerable to soil liquefaction during earthquakes, which stimulates seismic hazard.

On the other side, Iraq, located southeast of Turkey, shares a border with Turkey's southeastern area. Iraq is located on the northern section of the Arabian Plate and is limited to the north and east by the Bitlis-Zagros Fold and Thrust Belt, which is formed when the Eurasian and Arabian plates collide, causing increased seismic activity. The rest of the country is primarily located on the Arabian Platform, far from major plate borders. The Dead Sea fault system, a large left-lateral transform fault, forms the western boundary of the Arabian Platform, approximately 250 kilometres from Iraq's westernmost point [2]. However, Iraq recently experienced the effects of earthquake epicenters and has the potential for substantial seismic dangers in the future. The northern section of the country is most vulnerable to high-magnitude earthquakes over the next 50 years [1].

# Methodology

Calculate liquefaction with shear wave velocity depending on soil conditions and the magnitude of the earthquake by measuring three parameters CSR, CRR, and FS. LiquefyPro 5.5 is one of the CivilTech engineering applications established to examine the possibility of liquefaction in seismic conditions. The liquefaction resistance, as measured by seismic testing, was computed using the formula below, the equations below have major factors that are utilised to analyse the potential of liquefaction (PL) in this study. This study identifies LP values based on the categorization of (Appendix 1):   𝐹S = CRR	1

CSR

𝐶𝐶𝑅  =  𝑒𝑥𝑝 {[(0.0073.𝑉𝑠1)2.8011 −2.6168.𝐼𝑛(𝑀𝑤)−0.0099.1𝑛(σ𝑣𝑜)+0.0028.𝐹𝐶+0.4809.𝚯−1 (𝑃𝐿)	2

1.946

𝑃𝑎 0.25

σ

𝑉𝑠1 = 𝑉𝑠 (	)

𝑣

… … … 3

) ……… 4

L

P = Θ (− 0.0073.Vs12.8011−1946.In(CSR)2.6168(Mw)−0.0099.In(σvo)+0.0028.FC

0.4809

Where:

Pa: stress equal to 100 kPa Vs: shear wave velocity CRR: cyclic resistance ratio CSR: cyclic stress ratio

FS: factor safety

PL: probability of liquefaction

In general, Vs values in this study are derived from actual field studies rather than empirical correlations.

The engineering properties of the various strata of many geophysical and soil investigation reports for projects in Iraq, and the parameters are evaluated from field and laboratory test results of the available geophysical and geotechnical investigation reports collected from different sources.

# Data collection

Data from this study were collected at the regional level. Data from Turkey show two events: Mw: 7.4 1999 Izmit (PGA: 0.41 g), Mw: 7.0 1967 (max: 0.28 g). Mudurnu [16]. Furthermore, various places in Iraq have alternative earthquake scenarios that have been recorded in various locations across Iraqi land [15]. The peak ground acceleration has been estimated using the information in Fig. 1. The key objective of this research is to apply Random Force (RF) to the prediction of liquefaction using Vs measurements.

Figure 1 Probabilistic seismic hazard in Iraq with a 2% chance of exceedance in 50 years [1].

# Random forest (RF)

The Random Forest (RF) methodology is an intelligent recognition system based on statistical learning theory [5]. It uses ensemble learning algorithms to create numerous predictors, making it suitable for both classification and regression problems. Random Forest in this research process as classification which is a method of group learning that improves the accuracy and robustness of classification problems. During training, the algorithm generates a large number of decision trees and outputs the class that represents the mode of the categorization classes. Each decision tree in the random forest is built with a subset of the training data and a random selection of features, which adds diversity to the trees and makes the model more resilient and less prone to overfitting.

Given that liquefaction evaluation often yields two outcomes (liquefaction or non-liquefaction) the study adopts a classification tree approach. The goal is to investigate the correlation between the chance of liquefaction and shear wave velocity. The liquefaction classifier is classified as binary (just 1/-1). [20] to distinguish between two categories, such as liquefaction as 1 and non-liquefaction as -1.

# Result and Discussion

The input parameters employed include σ′v, soil type, Fc, Vs, PGA, and M to evaluate the Probability of Liquefaction area of study PL. The data presented in Table (Appendix 1) some areas exhibit no signs of liquefaction, while others demonstrate liquefaction potential. This finding is reassuring regarding seismic risk, as it indicates that, under ordinary conditions, there is less probability of the soil being damaged due to liquefaction. However, these results need to be interpreted with caution, considering the variations in susceptibility to liquefaction from one location to another and also due to different types of soils.

The research also highlights the importance of shear wave velocity (Vs). Generally, the firmer the soil (the higher the Vs value), the more effective it is at resisting liquefaction. The softer the soil - represented by a lower Vs value - the more liquefiable it is. Hence, sites with lower Vs values could be more prone to liquefaction even in cases without strong seismic action [14].

Peak ground acceleration (PGA) is one of the important measures in estimating liquefaction potential since it directly affects the dynamic loading received by the soil during an earthquake. Higher PGA values indicate more ground shaking, which can cause liquefaction in sensitive soils. Thus, even in places with relatively moderate earthquake magnitudes, elevated PGA levels might increase the risk of liquefaction, especially in areas with softer soil [16].

Furthermore, the magnitude of the earthquake (M) significantly influences liquefaction susceptibility. While the overall results show low magnitudes, it is vital to recognise that larger magnitude earthquakes can greatly increase liquefaction potential, particularly in places with susceptible soil conditions [7].

The evaluation of liquefaction by using the Random Forest algorithm RF. The general reliability of this model in predicting liquefaction appears to be good. The evaluation matrix shows that accuracy is 92.22%, precision is 93.5%, Recall: is 93.5%, and specificity is 90.3% with a high precision, which is a proportion of actual liquefaction cases among all predicted cases. Furthermore, the recall and specificity, which represent the percentage of actual liquefaction and non-liquefaction cases, demonstrate effective prediction of liquefaction based on shear wave velocity, given the high-performance metric observed.

Table 1 Statistical analysis of the dataset

|  | M | a max(g) | vs (m/s) | LP |
| --- | --- | --- | --- | --- |
| count | 219 | 219 | 219 | 219 |
| mean | 6.704762 | 0.347275 | 247.263004 | 0.003663 |
| std | 0.831688 | 0.079892 | 142.041713 | 1.00183 |
| min | 5 | 0.172 | 102 | -1 |
| 25% | 6 | 0.26 | 181 | -1 |
| 50% | 7.4 | 0.41 | 201 | 1 |
| 75% | 7.4 | 0.41 | 246 | 1 |
| max | 7.5 | 0.43 | 1203 | 1 |

# Conclusions

To sum up, our research has yielded significant insights into the evaluation of soil liquefaction potential in Turkey and Iraq through the application of a combination of geotechnical engineering and geophysical studies. It was observed that most of Iraq's regions were less likely to experience liquefaction because of incidents involving low seismic magnitudes and wide divisions from seismic sources; however, some areas were more susceptible because of structural weaknesses in the composition of the soil. This illustrated the importance of conducting comprehensive geotechnical studies in earthquake-prone areas in order to ensure infrastructure robustness and community safety.

However, with several high-magnitude earthquake epicentres, Turkey showed clear evidence of liquefaction, and thereby indicated the importance of risk mitigation methods. Our studies also showed that

machine learning, more specifically the Random Forest algorithm in the prediction of liquefaction potential using shear wave velocity. The high accuracy, precision, recall, and specificity in our evaluation metrics depict the model to be reliable in forecasting liquefaction events. These findings emphases the importance of adding advanced analytical tools to liquefaction assessments in view of increasing prediction precision and catastrophe management.

# Acknowledgements

The corresponding author would like to thank Dr Wisam R. Muttashar for his essential guidance, helpful recommendations, and unwavering support throughout the research process.

# References

- Abdulnaby, W., 2019. Structural geology and neotectonics of Iraq, Northwest Zagros. In Developments in structural geology and tectonics. Elsevier.

- Al-Taie, E., Al-Ansari, N. and Knutsson, S., 2014. The need to develop a building code for Iraq. Engineering, 6(10), pp.610-632.

- Andrus, R.D. and Stokoe, K.H., 1999. Liquefaction resistance based on shear wave velocity.

- Andrus RD, Stokoe KH II (2000) Liquefaction resistance of soils from shear wave velocity. J Geotech Geoenviron Eng 126(11):1015–1025.

- Breiman, L. (2001). Random forests. Machine learning, 45, 5-32.

[7] Bhutani, M., & Naval, S. (2020). Assessment of seismic site response and liquefaction potential for some sites using borelog data. Civ. Eng. J, 6, 2103-2119.

- Boulanger, R.W. and Idriss, I.M., 2012. Probabilistic standard penetration test–based liquefaction–triggering procedure. Journal of Geotechnical and Geoenvironmental Engineering, 138(10), pp.1185-1195.

- Boulanger, R.W. and Idriss, I.M., 2014. CPT and SPT-based liquefaction triggering procedures. Report No. UCD/CGM.- 14, 1.

- Castro G (1969) Liquefaction of sands, Harvard Soil Mechanics Series, 81. Pierce Hall, Cambridge,

- Cetin, K.O., Seed, R.B., Der Kiureghian, A., Tokimatsu, K., Harder Jr, L.F., Kayen, R.E. and Moss, R.E., 2004. Standard penetration test-based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of Geotechnical and Geoenvironmental Engineering, 130(12), p.1314.

- Idriss IM, Boulanger RW (2008) Soil liquefaction during earthquakes. Earthquake Engineering Research

- NCEER and 1998 NCEER/NSF workshops on evaluation of liquefaction resistance of soils. Journal of geotechnical and geoenvironmental engineering, 1. 2.-3., n.d. NCEER and 1998 NCEER/NSF workshops on evaluation of liquefaction resistance of soils. Journal of geotechnical and geoenvironmental engineering, 127(4), 297-313.

- Kennedy, S., & Mahmood, R. A. (2023). Liquefaction Potential Assessment for Basrah Soil Based on Standard Penetration Test Values. The Iraqi Geological Journal, 283-293.

- Kayen, R., Moss, R. E. S., Thompson, E. M., Seed, R. B., Cetin, K. O., Kiureghian, A. D., ... & Tokimatsu, K. (2013). Shear-wave velocity–based probabilistic and deterministic assessment of seismic soil liquefaction potential. Journal of Geotechnical and Geoenvironmental Engineering, 139(3), 407-419.

- Mohammed, Q. S. A. A. D., & Sa'ur, R. H. (2016). Database for dynamic soil properties of seismic active zones in Iraq. Journal of Engineering, 22(7), 1-18.

- Onur, T., Gok, R., Abdulnaby, W., Shakir, A.M., Mahdi, H., Numan, N., Al-Shukri, H., Chlaib, H.K., Ameen, T.H. and Abd, N.A., 2016. Probabilistic seismic hazard assessment for Iraq(No. LLNL-TR-691152). Lawrence Livermore National Lab. (LLNL), Livermore, CA (United States).

[16] Seed, H. a. I. I., 1971. Simplified procedure for evaluating soil liquefaction potential. Institute, EERI Publication, Monograph MNO-12, Oakland, CA.

- Silahtar, A., Karaaslan, H., & Kocaman, K. (2023). Site Characterization and Liquefaction Hazard Assessment for the Erenler Settlement Area (Sakarya Province, Turkey) Based on Integrated SPT-Vs Data. Sustainability, 15(2), 1534.

- Stokoe, K.H., Nazarian II, S., Rix, G.J., Sanchez-Salinero, I., Sheu, J.C., Mok, Y.J., 1988. In situ seismic testing of hard-to-sample soils by surface wave method. In: Von Thun, J.L. (Ed.), Earthquake Engineering and Soil Dynamics II—Recent Advances in Ground-Motion Evaluation, 20. Geotech. Spec. Publ., New York, pp. 264–289.

- Watanachaturaporn, P., Varshney, P. K., & Arora, M. K. (2004, May). Evaluation of factors affecting support vector machines for hyperspectral classification. In the American Society for Photogrammetry & Remote Sensing (ASPRS) 2004 Annual Conference, Denver, CO.

- Youd, T.L. and Idriss, I.M., 2001. Liquefaction resistance of soils: Summary report from the 1996 NCEER and 1998 NCEER/NSF workshops on evaluation of liquefaction resistance of soils. Journal of geotechnical and geoenvironmental engineering, 127(4), 297-313.

Appendix 1: Dataset used to calculate LP.

| M | a max(g) | vs (m/s) | FC(%) | 𝝈v' (kPa) | LP (actual) |
| --- | --- | --- | --- | --- | --- |
| 7.4 | 0.41 | 209 | 4 | 72 | 1 |
| 7.4 | 0.41 | 201 | 22 | 85 | -1 |
| 7.4 | 0.41 | 194 | 47 | 97 | -1 |
| 7.4 | 0.41 | 170 | 58 | 70 | -1 |
| 7.4 | 0.41 | 190 | 35 | 124 | -1 |
| 7.4 | 0.41 | 185 | 35 | 137 | -1 |
| 7.4 | 0.41 | 162 | 64 | 78 | -1 |
| 7.4 | 0.41 | 157 | 64 | 142 | -1 |
| 7.4 | 0.41 | 153 | 65 | 155 | -1 |
| 7.4 | 0.41 | 207 | 70 | 57 | 1 |
| 7.4 | 0.41 | 198 | 61 | 68 | -1 |
| 7.4 | 0.41 | 191 | 69 | 79 | -1 |
| 7.4 | 0.41 | 185 | 70 | 90 | -1 |
| 7.4 | 0.41 | 213 | 68 | 101 | 1 |
| 7.4 | 0.41 | 208 | 68 | 112 | -1 |
| 7.4 | 0.41 | 220 | 70 | 123 | 1 |
| 7.4 | 0.41 | 216 | 69 | 134 | 1 |
| 7.4 | 0.41 | 206 | 14 | 39 | -1 |
| 7.4 | 0.41 | 194 | 6 | 49 | -1 |
| 7.4 | 0.41 | 221 | 6 | 59 | 1 |
| 7.4 | 0.41 | 212 | 10 | 70 | -1 |
| 7.4 | 0.41 | 246 | 16 | 91 | 1 |
| 7.4 | 0.41 | 239 | 21 | 101 | 1 |
| 7.4 | 0.41 | 233 | 22 | 112 | 1 |
| 7.4 | 0.41 | 175 | 26 | 122 | -1 |
| 7.4 | 0.41 | 172 | 26 | 132 | -1 |
| 7.4 | 0.41 | 168 | 28 | 143 | -1 |
| 7.4 | 0.41 | 165 | 24 | 153 | -1 |
| 7.4 | 0.41 | 242 | 79 | 63 | 1 |
| 7.4 | 0.41 | 242 | 65 | 127 | 1 |
| 7.4 | 0.41 | 157 | 71 | 60 | -1 |
| 7.4 | 0.41 | 188 | 76 | 80 | -1 |
| 7.4 | 0.41 | 151 | 68 | 90 | -1 |
| 7.4 | 0.41 | 147 | 68 | 100 | -1 |
| 7.4 | 0.41 | 201 | 70 | 58 | -1 |
| 7.4 | 0.41 | 195 | 61 | 69 | -1 |
| 7.4 | 0.41 | 188 | 69 | 81 | -1 |
| 7.4 | 0.41 | 182 | 70 | 92 | -1 |
| 7.4 | 0.41 | 190 | 68 | 103 | -1 |
| 7.4 | 0.41 | 185 | 68 | 114 | -1 |
| 7.4 | 0.41 | 180 | 70 | 126 | -1 |
| 7.4 | 0.41 | 189 | 69 | 137 | -1 |
| 7.4 | 0.41 | 194 | 71 | 65 | -1 |
| 7.4 | 0.41 | 187 | 68 | 74 | -1 |
| 7.4 | 0.41 | 204 | 71 | 67 | -1 |
| 7.4 | 0.41 | 195 | 27 | 79 | -1 |
| 7.4 | 0.41 | 187 | 63 | 141 | -1 |
| 7.4 | 0.41 | 183 | 71 | 153 | -1 |
| 7.4 | 0.41 | 180 | 70 | 165 | -1 |
| 7.4 | 0.41 | 180 | 30 | 55 | -1 |
| 7.4 | 0.41 | 211 | 30 | 66 | -1 |
| 7.4 | 0.41 | 202 | 24 | 78 | -1 |
| 7.4 | 0.41 | 228 | 32 | 90 | 1 |

| 7.4 | 0.41 | 221 | 32 | 101 | 1 |
| --- | --- | --- | --- | --- | --- |
| 7.4 | 0.41 | 215 | 33 | 113 | 1 |
| 7.4 | 0.41 | 210 | 32 | 125 | -1 |
| 7.4 | 0.41 | 205 | 23 | 136 | -1 |
| 7.4 | 0.41 | 201 | 32 | 148 | -1 |
| 7.4 | 0.41 | 168 | 85 | 69 | -1 |
| 7.4 | 0.41 | 161 | 84 | 81 | -1 |
| 7.4 | 0.41 | 206 | 85 | 73 | 1 |
| 7.4 | 0.41 | 198 | 4 | 85 | -1 |
| 7.4 | 0.41 | 191 | 2 | 98 | -1 |
| 7.4 | 0.41 | 204 | 3 | 111 | -1 |
| 7.4 | 0.41 | 198 | 2 | 123 | -1 |
| 7.4 | 0.41 | 193 | 3 | 136 | -1 |
| 7.4 | 0.41 | 189 | 2 | 149 | -1 |
| 7.4 | 0.41 | 160 | 2 | 162 | -1 |
| 7.4 | 0.41 | 157 | 3 | 174 | -1 |
| 7.4 | 0.41 | 154 | 3 | 187 | -1 |
| 7.4 | 0.41 | 189 | 85 | 66 | -1 |
| 7.4 | 0.41 | 180 | 82 | 80 | -1 |
| 7.4 | 0.41 | 213 | 84 | 93 | 1 |
| 7.4 | 0.41 | 199 | 22 | 121 | -1 |
| 7.4 | 0.41 | 213 | 88 | 149 | 1 |
| 7.4 | 0.41 | 208 | 83 | 162 | 1 |
| 7.4 | 0.41 | 124 | 19 | 114 | -1 |
| 7.4 | 0.41 | 140 | 92 | 73 | -1 |
| 7.4 | 0.41 | 135 | 92 | 86 | -1 |
| 7.4 | 0.41 | 199 | 82 | 98 | 1 |
| 7.4 | 0.41 | 193 | 88 | 110 | -1 |
| 7.4 | 0.41 | 188 | 16 | 123 | -1 |
| 7.4 | 0.41 | 184 | 15 | 135 | -1 |
| 7.4 | 0.41 | 150 | 15 | 148 | -1 |
| 7.4 | 0.41 | 147 | 13 | 161 | -1 |
| 7.4 | 0.41 | 145 | 14 | 174 | -1 |
| 7.4 | 0.41 | 142 | 13 | 186 | -1 |
| 7.4 | 0.41 | 137 | 9.2 | 46 | -1 |
| 7.4 | 0.41 | 138 | 8 | 85 | -1 |
| 7.4 | 0.41 | 236 | 10 | 98 | 1 |
| 7.4 | 0.41 | 229 | 15 | 111 | 1 |
| 7.4 | 0.41 | 223 | 53 | 124 | 1 |
| 7.4 | 0.41 | 217 | 54 | 137 | 1 |
| 7.4 | 0.41 | 243 | 83 | 41 | 1 |
| 7.4 | 0.41 | 210 | 83 | 52 | 1 |
| 7.4 | 0.41 | 200 | 96 | 63 | -1 |
| 7.4 | 0.41 | 192 | 96 | 74 | -1 |
| 7.4 | 0.41 | 136 | 96 | 85 | -1 |
| 7.4 | 0.41 | 132 | 96 | 95 | -1 |
| 7.4 | 0.41 | 128 | 96 | 106 | -1 |
| 7.4 | 0.41 | 170 | 34 | 74 | -1 |
| 7.4 | 0.41 | 213 | 37 | 62 | 1 |
| 7.4 | 0.41 | 199 | 40 | 73 | -1 |
| 7.4 | 0.41 | 211 | 34 | 105 | -1 |
| 7.4 | 0.41 | 206 | 34 | 116 | -1 |
| 7.4 | 0.41 | 180 | 51 | 59 | -1 |
| 7.4 | 0.41 | 171 | 52 | 73 | -1 |
| 7.4 | 0.41 | 152 | 55 | 99 | -1 |
| 7.4 | 0.41 | 148 | 70 | 113 | -1 |
| 7.4 | 0.41 | 195 | 71 | 126 | -1 |
| 7.4 | 0.41 | 237 | 3 | 69 | 1 |
| 7.4 | 0.41 | 227 | 15 | 81 | 1 |
| 7.4 | 0.41 | 220 | 13 | 94 | 1 |
| 7.4 | 0.41 | 193 | 12 | 106 | -1 |
| 7.4 | 0.41 | 188 | 9 | 118 | -1 |
| 7.4 | 0.41 | 176 | 23 | 82 | -1 |
| 7.4 | 0.41 | 174 | 15 | 117 | -1 |
| 7.4 | 0.41 | 184 | 27 | 130 | -1 |
| 7.4 | 0.41 | 179 | 21 | 142 | -1 |

| 7.4 | 0.41 | 176 | 16 | 155 | -1 |
| --- | --- | --- | --- | --- | --- |
| 7.4 | 0.41 | 194 | 93 | 51 | 1 |
| 7.4 | 0.41 | 197 | 93 | 64 | -1 |
| 7.4 | 0.41 | 188 | 93 | 77 | -1 |
| 7.4 | 0.41 | 181 | 93 | 90 | -1 |
| 7.4 | 0.41 | 211 | 17 | 103 | -1 |
| 7.4 | 0.41 | 204 | 16 | 116 | -1 |
| 7.4 | 0.41 | 199 | 19 | 129 | -1 |
| 7.4 | 0.41 | 212 | 93 | 142 | 1 |
| 7.4 | 0.41 | 207 | 93 | 155 | 1 |
| 7.4 | 0.41 | 203 | 93 | 168 | -1 |
| 7.4 | 0.41 | 199 | 93 | 181 | -1 |
| 7.4 | 0.41 | 196 | 93 | 194 | -1 |
| 7.4 | 0.41 | 179 | 28 | 67 | -1 |
| 7.4 | 0.41 | 197 | 24 | 95 | -1 |
| 7.4 | 0.41 | 207 | 8 | 135 | -1 |
| 7.4 | 0.41 | 184 | 43 | 84 | -1 |
| 7.5 | 0.34 | 173 | 9 | 62.1 | -1 |
| 7.5 | 0.34 | 176 | 11 | 77 | -1 |
| 7.5 | 0.34 | 181 | 14 | 92.3 | -1 |
| 7.5 | 0.34 | 191 | 13 | 107.6 | -1 |
| 7.5 | 0.34 | 191 | 12 | 122.9 | -1 |
| 7.5 | 0.34 | 194 | 14 | 138.8 | -1 |
| 7.5 | 0.34 | 200 | 15 | 154.7 | -1 |
| 6.5 | 0.23 | 175 | 4 | 62.4 | 1 |
| 6.5 | 0.23 | 174 | 8 | 77.3 | -1 |
| 6.5 | 0.23 | 179 | 7 | 92.5 | -1 |
| 6.5 | 0.23 | 180 | 4 | 107.6 | -1 |
| 6.5 | 0.23 | 191 | 4 | 122.8 | -1 |
| 6.5 | 0.23 | 193 | 4 | 138.7 | -1 |
| 6.5 | 0.23 | 201 | 5 | 154.6 | -1 |
| 7.5 | 0.3 | 175 | 5 | 140.6 | -1 |
| 7.5 | 0.3 | 196 | 8 | 154.4 | -1 |
| 7.5 | 0.3 | 224 | 12 | 168.3 | -1 |
| 7.5 | 0.3 | 238 | 10 | 182.5 | 1 |
| 7.5 | 0.3 | 240 | 7 | 196.7 | 1 |
| 7.5 | 0.3 | 241 | 6 | 210.9 | 1 |
| 6.5 | 0.24 | 175 | 6 | 141.4 | -1 |
| 6.5 | 0.24 | 192 | 8 | 155.3 | -1 |
| 6.5 | 0.24 | 223 | 11 | 169.2 | -1 |
| 6.5 | 0.24 | 221 | 10 | 183 | -1 |
| 6.5 | 0.24 | 231 | 7 | 197.1 | -1 |
| 6.5 | 0.24 | 242 | 6 | 211.4 | 1 |
| 6.3 | 0.4 | 302 | 45 | 35.6 | 1 |
| 6.3 | 0.4 | 468 | 10 | 109.8 | 1 |
| 6.3 | 0.4 | 832 | 8 | 58.8 | 1 |
| 6.3 | 0.4 | 274 | 50 | 52.2 | 1 |
| 6.3 | 0.4 | 354 | 45 | 121.2 | 1 |
| 6.3 | 0.4 | 260 | 10 | 28.2 | 1 |
| 6.3 | 0.4 | 296 | 30 | 74.6 | 1 |
| 6.3 | 0.4 | 462 | 33 | 159 | 1 |
| 6.1 | 0.43 | 262 | 0 | 50.2 | 1 |
| 6.1 | 0.43 | 576 | 0 | 201.7 | 1 |
| 6.1 | 0.43 | 384 | 0 | 68.8 | 1 |
| 6.1 | 0.43 | 233 | 0 | 68.8 | 1 |
| 6.3 | 0.26 | 219 | 38 | 49.4 | 1 |
| 6.3 | 0.26 | 301 | 10 | 69 | 1 |
| 6.3 | 0.26 | 733 | 10 | 206 | 1 |
| 6.3 | 0.26 | 145 | 20 | 29 | 1 |
| 6.3 | 0.26 | 212 | 60 | 59 | 1 |
| 6.3 | 0.26 | 323 | 60 | 184.2 | 1 |
| 6.3 | 0.26 | 225 | 18 | 32.8 | 1 |
| 6.3 | 0.26 | 321 | 60 | 173.2 | 1 |
| 6.3 | 0.26 | 476 | 0 | 284.5 | 1 |
| 6.3 | 0.26 | 304 | 66 | 121.8 | 1 |
| 6.3 | 0.26 | 312 | 65 | 112.8 | 1 |

| 6.2 | 0.43 | 459 | 66 | 147.8 | 1 |
| --- | --- | --- | --- | --- | --- |
| 6.2 | 0.43 | 319 | 22 | 73.55 | 1 |
| 6.2 | 0.43 | 348 | 5 | 163.55 | 1 |
| 5.2 | 0.36 | 303 | 10 | 51 | 1 |
| 5.2 | 0.36 | 362 | 10 | 76.2 | 1 |
| 5.2 | 0.36 | 292 | 10 | 26.8 | 1 |
| 5.2 | 0.36 | 346 | 15 | 57.7 | 1 |
| 6 | 0.3 | 606 | 10 | 134.3 | 1 |
| 6 | 0.3 | 451 | 6 | 44.2 | 1 |
| 6 | 0.3 | 701 | 5 | 117.8 | 1 |
| 6 | 0.3 | 186 | 28 | 98.8 | 1 |
| 6 | 0.3 | 258 | 9 | 160 | 1 |
| 6 | 0.3 | 265 | 60 | 92.2 | 1 |
| 6 | 0.3 | 395 | 30 | 157.3 | 1 |
| 6 | 0.3 | 153 | 30 | 98.8 | -1 |
| 6 | 0.3 | 215 | 25 | 172.4 | 1 |
| 6 | 0.3 | 298 | 50 | 108.8 | 1 |
| 6 | 0.3 | 428 | 45 | 160.8 | 1 |
| 6 | 0.3 | 507 | 10 | 413.25 | 1 |
| 6 | 0.3 | 240 | 25 | 110.8 | 1 |
| 6 | 0.3 | 430 | 10 | 153.8 | 1 |
| 6 | 0.3 | 140 | 20 | 19 | 1 |
| 6 | 0.3 | 219 | 50 | 146.12 | 1 |
| 6 | 0.3 | 408 | 10 | 227.67 | 1 |
| 6 | 0.3 | 189 | 30 | 69.8 | 1 |
| 6 | 0.3 | 248 | 60 | 125 | 1 |
| 6 | 0.3 | 225 | 10 | 271.4 | 1 |
| 6 | 0.3 | 165 | 60 | 84.8 | -1 |
| 6 | 0.3 | 279 | 10 | 126.2 | 1 |
| 6 | 0.3 | 260 | 50 | 67.1 | 1 |
| 6 | 0.31 | 369 | 10 | 155.1 | 1 |
| 6 | 0.31 | 111 | 30 | 22.56 | -1 |
| 6 | 0.172 | 183 | 20 | 134.88 | 1 |
| 6 | 0.172 | 372 | 45 | 122.8 | 1 |
| 6 | 0.172 | 398 | 45 | 167.3 | 1 |
| 5 | 0.2 | 817.8 | 20 | 54.3 | 1 |
| 5 | 0.2 | 1164 | 10 | 117.3 | 1 |
| 5 | 0.2 | 1203 | 5 | 240.8 | 1 |
| 5 | 0.2 | 257 | 50 | 62.8 | 1 |
| 5 | 0.2 | 379 | 14 | 104.8 | 1 |
| 5 | 0.2 | 198 | 50 | 58.3 | 1 |
| 5 | 0.2 | 265 | 20 | 92.7 | 1 |
| 5 | 0.2 | 497 | 15 | 246 | 1 |
| 5 | 0.2 | 417 | 35 | 186.2 | 1 |
| 5 | 0.2 | 312 | 30 | 101.15 | 1 |
| 5 | 0.2 | 289 | 0 | 142.85 | 1 |
| 5 | 0.2 | 284 | 15 | 50.3 | 1 |
| 5 | 0.2 | 550 | 15 | 37.7 | 1 |
| 6 | 0.37 | 563 | 18 | 199.1 | 1 |
| 6 | 0.37 | 618 | 3 | 86.8 | 1 |
| 6 | 0.37 | 268 | 15 | 19.1 | 1 |
| 6 | 0.37 | 557 | 5 | 58.2 | 1 |
| 6 | 0.37 | 659 | 23 | 148.8 | 1 |
| 6 | 0.37 | 111 | 50 | 66.2 | -1 |
| 6 | 0.37 | 152 | 10 | 151.4 | -1 |
| 6 | 0.37 | 211 | 50 | 327.2 | 1 |
| 6 | 0.37 | 131 | 50 | 50.8 | -1 |
| 6 | 0.37 | 250 | 50 | 79.9 | 1 |
| 6 | 0.37 | 420 | 50 | 268.9 | 1 |
| 5.5 | 0.2 | 179 | 50 | 98.9 | 1 |
| 5.5 | 0.2 | 380 | 50 | 198.8 | 1 |
| 5.5 | 0.2 | 176 | 50 | 53.45 | 1 |
| 5.5 | 0.2 | 200 | 10 | 53.25 | 1 |
| 5.5 | 0.2 | 250 | 60 | 145.8 | 1 |
| 5.5 | 0.2 | 225 | 40 | 58.8 | 1 |
| 5.5 | 0.2 | 243 | 15 | 69.7 | 1 |

| 5.5 | 0.2 | 333 | 60 | 205.7 | 1 |
| --- | --- | --- | --- | --- | --- |
| 5.5 | 0.2 | 188 | 60 | 25.1 | 1 |
| 5.5 | 0.2 | 185 | 10 | 30.2 | 1 |
| 5.5 | 0.2 | 200 | 60 | 136.2 | 1 |
| 5.5 | 0.2 | 185 | 30 | 84.2 | 1 |
| 5.5 | 0.2 | 321 | 50 | 121.8 | 1 |
| 5.5 | 0.25 | 110 | 60 | 126.2 | -1 |
| 5.5 | 0.25 | 145 | 15 | 148.2 | 1 |
| 5.5 | 0.25 | 166 | 60 | 393.2 | 1 |
| 5.5 | 0.25 | 200 | 60 | 46.88 | 1 |
| 5.5 | 0.25 | 240 | 60 | 107.66 | 1 |
| 5.5 | 0.25 | 329 | 15 | 41.3 | 1 |
| 5.5 | 0.25 | 627 | 10 | 105 | 1 |
| 5.5 | 0.25 | 230 | 3 | 36.8 | 1 |
| 5.5 | 0.25 | 365 | 15 | 129.96 | 1 |
| 5.5 | 0.25 | 166 | 50 | 77.6 | 1 |
| 5.5 | 0.25 | 194 | 10 | 160.4 | 1 |
| 5.5 | 0.25 | 117 | 50 | 69.2 | -1 |
| 5.5 | 0.25 | 198 | 50 | 135.8 | 1 |
| 5.5 | 0.25 | 138 | 60 | 95.5 | -1 |
| 5.5 | 0.25 | 103 | 15 | 124.99 | -1 |
| 5.5 | 0.25 | 102 | 50 | 329.13 | -1 |