Animal nutrition practical second stage (( 2024-2025)

---------------------------------------------------------------------------

DETERMINATION OF MOISTURE

Moisture content is one of the most commonly measured properties of

food materials. It is important to food scientists for a number of different reasons:

- Legal and Labeling Requirements. There are legal limits to the maximum or minimum amount of water that must be present in certain types of food.

- Economic. The cost of many foods depends on the amount of water they contain - water is an inexpensive ingredient, and manufacturers often try to incorporate as much as possible in a food, without exceeding some maximum legal requirement.

- Microbial Stability. The propensity of microorganisms to grow in foods depends on their water content. For this reason many foods are dried below some critical moisture content.

- Food Quality. The texture, taste, appearance and stability of foods depends on the amount of water they contain.

- Food Processing Operations. A knowledge of the moisture content is often necessary to predict the behavior of foods during processing, e.g. mixing, drying, flow through a pipe or packaging.

It is therefore important for food scientists to be able to reliably measure moisture contents. A number of analytical techniques have been developed for this purpose, which vary in their accuracy, cost, speed, sensitivity, specificity, ease of operation, etc. The choice of an analytical procedure for a particular application depends on the nature of the food being analyzed and the reason the information is needed.

Properties of Water in Foods

The moisture content of a food material is defined through the following equation:

%Moisture = (mw/m sample) 100

Where mw is the mass of the water and msample is the mass of the sample.

In principle:-

the moisture content of a food can therefore be determined accurately by measuring the number or mass of water molecules present in a known mass of sample. It is not possible to directly measure the number of water molecules present in a sample because of the huge number of molecules involved. A number of analytical techniques commonly used to determine the moisture content of foods are based on determinations of the mass of water present in a known mass of sample. Nevertheless, as we will see later, there are a number of practical problems associated with these techniques that make highly accurate determinations of moisture content difficult or that limit their use for certain applications. For these reasons, a number of other analytical methods have been developed to measure the moisture content of foods that do not rely on direct measurement of the mass of water in a food. Instead, these techniques are based on the fact that the water in a food can be distinguished from the other components in some measurable way.

the water molecules in a food may be present in a variety of different molecular environments depending on their interaction with the surrounding molecules.�

The water molecules in these different environments normally have different physiochemical properties:

- Bulk water.

- Capillary or trapped water.

- Physically bound water..

- Chemically bound water..

Determination of moisture in feed sample:

- The aim of this experiment:

- Prepare the samples for chemical analysis and report the chemical analysis on a dry basis.

- Determination of moisture occur to know the spoilage of feed.

- Determination price of forage depend on percentage of moisture

Scientific principles of experiment:

The speed of water evaporation from feed surface depend on the vapor pressure of water and temperature.

The partial vapor pressure of water outside feed should be less than the partial vapor pressure of water in feed to facility remove moisture from feed to outer environment.

Note: the normal value of moisture in concentrate about (3%-20%) while the normal value of moisture in green roughage about (75%-80%).

Equipment and machines used:

- Small basin used for concentrate.

- Large basin used for green roughage.

- 3-Oven.

- Desiccators(contain CaCl2)

- Electrical balance

Method

A-Determination of moisture in concentrates.

- Put the clean small basin in oven with 105 ºC for 1hr.

- Remove the small basin from oven to the desiccators until cool then weight & record it weight when it empty.

- Weight about (1-2) gram in to a weight basin.

- Put the basin contain the sample in oven with 105 ºCover night or in 135 ºC for 3 hrs.

- After the end of drying period we removed the basin from oven to desiccators until cool .remove from desiccators and weight as quickly as possible.

- Calculation of percentage of moisture in sample by

Wt. of moisture = wt.of basin with sample before drying –wt. of basin with sample after drying.

B- Determination of moisture in green roughage :

- Put clean large basin in oven with 105c for 1hr.

- Remove the basin from oven into desiccators until cool then records its weight by using electrical balance.

- Weight	10 gram from green roughage  (hay,  arge basin.

- Put the sample in oven with 60º C for 24 hrs.

- Remove the basin from oven and put it in desiccators until cool then record the weight of sample after drying.

Note: this moisture called primary moisture in green roughage

Determination of secondary moisture in green roughage after determination of primary moisture grained the drying sample then take 1-2 gm from it. then determinate the secondary moisture by the same steps of determination of moisture by the same steps of determination of moisture in concentrate diet.

%dry matter=100-%of moisture

C- Determination of moisture in meat :

- Put clean large basin in oven with 105c for 1hr.

- Remove the basin from oven into desiccators until cool then records its weight by using electrical balance.

- Weight 20-25 gram from meat ( cutting into small pieces &put it in large basin).

- Put the sample in oven with 105 º C for 3 hrs.

- Remove the basin from oven and put it in desiccators until cool then record the weight of sample after drying.

.

Note: the moisture percentage differ in food depend on kinds of foods for example: