Determination of Lead (Pb) in Water using FAAS

Calibration Standards

| Pb (mg/L) | Absorbance (A) |
| --- | --- |
| 0.0 | 0.005 |
| 0.5 | 0.105 |
| 1.0 | 0.205 |
| 2.0 | 0.405 |

Sample Absorbance: A = 0.255

Step 1: Calibration Curve Equation

A = m * C + b

- Slope (m): (0.205 - 0.105) / (1.0 - 0.5) = 0.200

- Intercept (b): 0.005

- Calibration equation: A = 0.200 * C + 0.005

Step 2: Solve for Sample Concentration

0.255 = 0.200 * C + 0.005

0.250 = 0.200 * C

C = 0.250 / 0.200 = 1.25 mg/L

Answer: Pb concentration in the water sample = 1.25 mg/L

Q2/

A water sample is analyzed for lead (Pb) using FAAS.
A 10.0 mL aliquot of the sample is diluted to 100.0 mL in a volumetric flask before measurement.
The following calibration data were obtained:

| Pb (mg/L) | Absorbance (A) |
| --- | --- |
| 0.00 | 0.000 |
| 0.50 | 0.110 |
| 1.00 | 0.210 |
| 2.00 | 0.410 |
| 3.00 | 0.610 |

The absorbance of the diluted sample is A = 0.315.

…………………………………………………………………………………………………………………………………………………………………